Usage Requirements

1. The motor driver must support the EtherCAT bus.

2. A bus controller is required.

3. Zdevelop version should be 2.5 or above.

Basic Parameters Configuration

The following parameters need to be set in the program, then motor can run normally.

- slot_scan(0), controller bus scan, this means it scans slot 0 (EtherCAT interface number, default 0). The scan returns -1 when scans successfully, this can be viewed through “?return”.

- axis_address(i)=1, select the first connected Ecat drive, and map it to axis i.

atype(i)=65, set the selected driver as axis i (65 means pulse type, 66 means speed closed loop, 67 means torque closed loop, set according to the driver control mode).

- slot_start(0), start slot button 0 bus, return -1 after successful start.

- base(i)

axis_enable=1, enable axis i.

- wdog=1, all axes are enabled.

The above 5 parameters are the basic instructions that must be included in the program. Some drives can already operate the motor at this time, such as Maxison's EP3 servo. Some need to continue to write the following instructions.

- DRIVE_PROFILE(i)=value, the configuration selection of the sending pdo and receiving pdo of each axis, generally the default is 0, some drives need to be set, such as Qingde Nengchuang, please consult the manufacturer for detailed configuration.

- DRIVE_CONTROLWORD=value, the drive control word, for the operation of the drive, the value of “value” corresponds to the function code in the drive manual.

For example:

IO Expansion

- The bus controller has a dedicated EIO expansion board, which is connected with a network cable through the EtherCAT interface, and the initial IO number is set by node_io(0,n)=value (n is the device No., 0 by default, and incremented by 1).

- The IO number of the expansion board increases by 16. Device 0:32, Device 1:48, Device 2:64, ...

- The node_io setting value will only be a multiple of 8. When it is set to 35, it will be changed to 32. For example, node_io(0,0)=50, device 0 48, device 1 48, device 2 64 (other devices will not change)

- The input and output numbers are consistent.

- The bus controller can also use ordinary ZIO expansion boards, and the method of use is the same as that of non-bus controllers.

Attentions

1. The device No. is incremented in order of 0 according to the connection sequence, as long as it supports the EtherCAT bus.

2. The axis No. of the pulse axis on the bus controller is fixed. The bus can also call the axis No. of pulse axis, when atype=65, the bus calls, atype! =65, it is in non-bus control. Note that dpos will change when switching, so it is best not to make axis Number conflict.

3. After clearing the driver alarm with driver_controlword (axis number), you need to delay for 200ms, and then use datum(0) to clear the controller alarm. If there is no delay, you may need to download the program twice to clear the alarm.

4. If the driver_controlword (axis number) is used to operate the driver, then all drivers after this driver will inherit this setting. So it's best to set it once per drive.

5. If you connect or disconnect a device during operation, you need to re-scan to update the status. For example, node_count(0) returns the number of currently connected devices, and the returned number will change after re-scanning.

6. axis_address(i)=1, select the first drive, not the first device.

For example, after connecting two expansion boards and then connecting two drives, the device numbers are expansion board a: 0, expansion board b: 1, drive a: 2, and drive b: 3.

At this time, axis_address(i)=1 selects drive a, and axis_address(i)=2 selects drive b.

Choose consecutively in order, you can’t choose 2 first and then choose 1, you can’t choose 1, skip 2 and choose 3.