This routine mainly shows window types and operations of hmi.

Simulator input “in” port functions:

| Press | Function | Release | Function |
| --- | --- | --- | --- |
| 0 | Switch to base window 2 | 0 | / |
| 1 | Switch to base window 1 | 1 | / |
| 2 | Open pop window 1 | 2 | Close pop window 1 |
| 3 | Open pop window 2 | 3 | Close pop window 2 |
| 4 | Open top window 1 | 4 | Close top window 1 |
| 5 | Open top window 2 | 5 | Close top window 2 |
| 6 | Open menu window 1 | 6 | Close menu window 1 |
| 7 | Open menu window 2 | 7 | Close menu window 2 |

Window can be called and closed through program instruction and functional keys.

When operating, please make sure window type and operation type is consistent, otherwise, it cannot achieve.

When operating, menu window belongs to pop type, the difference is menu window will close automatically.

Base Window:

New built windows are base window by default.

One base window must be as bottom window for hmi display.

It can only show one base window.

Base window is switched through program or object operation.

Keyboard Window

It is used for value and string objects those need to input.

Pop Window:

Pop window is similar to dialog box, which pops up window dynamically, and it shows the last called pop window according to calling sequence.

When multiple pop windows overlap, the function keys in the pop windows will be triggered normally.

When pop window is called, it can operate base window or other types.

Pop window is closed through program or object operation, when window is switched, also will be off.

Menu Window:

Menu window is similar to pop window, they will both pop up after called.

However, when Menu window popped up, at this time, it only can operate Menu window.

When click non-Menu window area, Menu window will close.

Top Window:

It always shows in the front, and usually it is a small window to achieve tool bar.

When switching window, Top window still shows the beginning, it will not close.

When there are multiple Top windows, showing according to calling sequence.

Top window is closed through program or object operation.

There are  two ways to show for Top window, one is showing at the beginning of program, another is showing by manually calling.

Showing at the beginning of the program: click “ Edit” – “hmi setting” to select initial Top window.

Showing by calling manually: through program or operating object.