Robotic Arm Document:

Robotic arm type: FRAME 2 / 12 – Delta (R type controllers support)

- Connection Configuration

  - ZDEVELOP Software

    - Simulator Imitate

- Open below selected .zpj file in ZDevelop.

- Click “Controller” – “Connect to simulator”.

- After connected, download the program into RAM/ROM, the program will execute automatically.

- Click “显示“ in simulator, then enter HMI interface.

    - Controller

Open project file, connect to controller, download into RAM/ROM (note ZMC0XX series only have ROM).

- Usage

  - Main Interface

Display part:

- Current State: Inverse mode or forward mode.

- Axis State: show motion state of current axis

Buttons:

- Axis Para Config (axis parameters setting): jump to the motion axis interface to set various basic motion parameters

- Robot Para Config (robotic arm parameters setting): jump to the robotic arm interface to set the parameters of the robot TABLE table

- Motion Mode: jump to the mode interface to choose inverse mode or forward mode.

- Inverse Kinematics: jump to the inverse mode interface, if the inverse solution motion mode is not selected, this button operation is invalid

- Manual: jump to the manual interface, if no motion mode is selected, this button operation is invalid.

  - Axis Para Config

- It can check basic initialization parameters of axis. Pulses / UNITS (pulse amount), SPEED (motion speed), atype (axis function), ACCEL (acceleration ) and DECEL (deceleration). At the same time, it can set and modify values.

- Save: when setting parameters or modifying parameters, click “Save”, so that after power-on, the set parameters are read instead of initialization parameters.

- “X” in upper right corner: it means return, after set or modified parameters, click this button, it will return to main interface.

  - Robot Para Config

  - Motion Mode

There are two modes:

- Inverse Kinematics: after clicking “Inverse Kinematics” button, it will automatically enter selection state under this inverse mode.

Smooth:

in this state, joint-axis uses owner speed and acceleration to do speed planning, but it may be out of shape in high-speed.

Forced:

in this state, joint-axis plans according to speed and acceleration of virtual-axis totally.

- Forward Kinematics: after clicking “Forward Kinematics” button, it will automatically enter forward state.

  - Inverse Kinematics

When clicking the inverse kinematics motion on the main interface, the premise is to enter the motion mode interface and select the inverse kinematics mode. Otherwise, there will be a prompt to select the inverse motion mode.

If you select the inverse kinematics mode on the motion mode interface, you will automatically enter the inverse kinematics mode interface when you click the inverse kinematics mode

- Actual axis and virtual axis: it shows current DPOS (demand/target position) from axis 0 to axis 7.

- Motion Distance: set motion distance, the inverse kinematics is used on joint-axis, and the joint-axis follows with virtual-axis. Therefore, parameters are positions to reach for axis 4 to axis 7.

- Current Mode: it shows current motion mode.

- Running: run virtual-axis.

- Stop: stop all axes.

- Clear: clear all current positions.

- Return (X): return to main interface, at this time, if you want to enter this interface, click “Inverse Kinematics” on main interface.

  - Manual

Before clicking Manual, you must select motion mode well, otherwise following will appear:

When motion mode is selected well, enter manual state. Then, manual button on main interface is valid.

- Left, Right: click manually to make current axis turn to left or turn to right.

- DPOS: it shows current axis position state.

- Distance: manually input the distance to move of current axis.

- Relative / Absolute: after entering the distance, it is relative or absolute motion.

- Move: when former all are set well, it is timer to move.

- Stop: clear current axis motion.

- Para Config: set axis parameters.

- Return: return to motion mode selection interface.

- Note:

Under forward kinematics manual, it cannot operate virtual-axis, and it cannot operate relative or absolute.

Under inverse kinematics manual, it cannot operate actual-axis.